# Spatial and temporal discretization of coupled generalized thermoelasticity: a measured comparison and an exact second-sound benchmark for layered cylinders

Authors. [Author 1]ᵃ, [Supervisor 1]ᵃ, [Supervisor 2]ᵃ ᵃ Department of Mechanical Engineering, Persian Gulf University, Bushehr, Iran

Author order to be settled with the supervisors. Add the thesis-derivation footnote. Cite Paper 1 for the formulation.

Target journal. Applied Mathematical Modelling (alternates: Computers & Structures, Engineering with Computers)

## Abstract

Coupled generalized thermoelasticity produces a stiff, strongly heterogeneous system in which a hyperbolic thermal wave is carried alongside an essentially undamped elastic field, and the numerical choices made in solving it are usually asserted rather than measured. This paper reports a like-for-like comparison of five spatial discretizations and six time integrators on identical problems with known exact solutions, together with an exact benchmark that other second-sound codes can reuse. Differential quadrature on a Chebyshev grid reaches the temporal error floor at roughly 9–11 points per direction, where quadratic finite elements need about 21 and both linear finite elements and second-order finite differences need about 161 — a fifteen-fold reduction per direction, squared in the plane. The same study exposes a limitation rarely emphasized: the differential quadrature error does not continue to fall but degrades and eventually breaks down under further refinement, catastrophically so on a uniform grid, so the method has a usable window rather than a convergence curve. Among the time integrators, Newmark, Houbolt and HHT-α are of equal accuracy at equal step, Wilson-θ pays a five-fold error penalty for its added dissipation, and the adaptive stiff solver buys one decimal digit at eight times the cost. The Laplace-transform route is shown to fail structurally on the fully coupled system, not merely to converge slowly: the undamped elastic poles of the coupled operator lie on the Bromwich inversion contour, so the numerical inversion is poisoned by near-singular solves — which explains why transform methods appear in this literature only when applied to the thermal subsystem. Finally, an exact Fourier–Bessel eigenfunction solution for the Lord–Shulman wave in a hollow cylinder is derived and independently verified (steady limit to 1.9 × 10⁻¹⁶, hyperbolic causality to 4 × 10⁻⁶, front-jump ray law to 0.16 %), then used to measure a layerwise differential quadrature solver at 0.32 % pointwise error.

Keywords. Generalized thermoelasticity; differential quadrature; time integration; Laplace inversion; second sound; benchmark solution

## 1. Introduction

[Target ≈ 900 words.]

Paragraph 1 — what makes this system hard. Coupled hyperbolic thermoelasticity carries two very different physical timescales in one system: a thermal wave that is damped and relatively slow, and an elastic field that is fast and essentially undamped. Spatial discretization produces a stiff ordinary differential system whose eigenvalue spectrum spans many orders of magnitude, and the boundary and interface conditions enter as algebraic constraints, so the semi-discrete problem is a differential-algebraic system rather than a pure ODE. Row scales differ by ten orders of magnitude between the thermal, mechanical and constraint equations.

Paragraph 2 — what the literature does. Differential quadrature is widely used for this class of problem and is routinely asserted to be more efficient than finite elements or finite differences, usually without a controlled measurement on the same problem with the same time march. Newmark is the default integrator, again largely by convention. Laplace transform methods appear regularly, but — a point this paper takes up — essentially always applied to the thermal subsystem alone.

Paragraph 3 — verification practice. Second-sound codes are frequently verified qualitatively, by observing that a front appears and moves at about the right speed. Exact references for the hyperbolic problem in a hollow cylinder are not readily available, and digitizing figures from earlier papers is the common substitute.

Paragraph 4 — contribution.

- A controlled comparison of five spatial discretizations on an identical problem with an exact solution and an identical time march, reporting the node count each needs to reach the temporal error floor.

- The observation that differential quadrature has a usable window rather than a monotone convergence curve, with measured breakdown on both Chebyshev and uniform grids.

- A comparison of six time integrators on the same system, including cost.

- A structural — not merely practical — explanation of why the Laplace route fails on the fully coupled operator, supported by two independent measured failures.

- An exact, independently verified Fourier–Bessel benchmark for the Lord–Shulman wave in a hollow cylinder, offered for reuse.

## 2. Model problem and semi-discrete system

Cite Paper 1 for the physical formulation; state here only what the numerical study needs.

The semi-discrete system is

Mx+Cx+Kx=F(t)

with x collecting nodal temperatures and the two displacement components. Rows arising from boundary and interface conditions are algebraic, with zero mass and damping entries, so the system is an index-1 differential-algebraic one. The thermal rows scale as ∼103, the mechanical rows as ∼1013 and the constraint rows as ∼1; each row is therefore equilibrated by its largest coefficient before factorization, which reduces the condition number of the effective matrix by several orders of magnitude and is a precondition for any of the comparisons that follow being meaningful.

[Figure 1: node layout and DOF numbering — figures_ch123/DQM_nodes_schematic.png.]

## 3. An exact benchmark for the Lord–Shulman wave

### 3.1 Construction

The benchmark is constructed so that it can be compared against the same solver configuration rather than against an idealized separate problem. Disabling the coupling term removes the only temperature-to-displacement feedback, so the energy row becomes exactly

ρc (θ+τ0θ)=kθ,rr+θ,r/r

Taking constant layer properties and an axially uniform inner boundary condition with insulated ends reduces the two-dimensional field to a one-dimensional radial problem.

That problem is solved in closed form by a Fourier–Bessel eigenfunction expansion. The time-dependent Dirichlet datum is lifted with the steady profile φ=a+blnr, which satisfies ∇2φ=0 exactly, and each modal amplitude then obeys a damped oscillator equation with exponential forcing that is solved analytically. There is consequently no time discretization anywhere in the reference and no numerical transform inversion; the only truncation is the number of retained modes.

### 3.2 Verification of the reference itself

A reference is only useful if it has been checked independently of the code it will be used to judge. Four checks were applied:

| check | result |
| --- | --- |
| Steady-state limit | 1.9 × 10⁻¹⁶ of ΔT |
| Strict hyperbolic causality (field ahead of the front) | 4 × 10⁻⁶ of ΔT |
| Front-jump ratios against the analytic ray law Δ⋅Ri/r e−(r−Ri)/2Cτ0 | 0.16 % |
| Fourier limit τ0→0 against an independent Crank–Nicolson solve | agreement |
| Self-convergence at 400 modes | 8 × 10⁻⁴ relative |

### 3.3 Measured solver accuracy

Against this reference the layerwise differential quadrature solver attains a maximum pointwise error of 0.32 % of ΔT and an L₂ error of 0.065 % on the finest mesh at the final time, with errors falling monotonically under refinement (L∞ 1.34 % → 0.32 %, L₂ 0.49 % → 0.065 % for 49 → 147 radial points). The measured front position tracks the exact front to within one grid spacing at every snapshot. The premise of the comparison is verified in-run: the solver’s two-dimensional field is axially uniform to 3.9 × 10⁻¹¹ of ΔT, confirming that it really did collapse to the one-dimensional problem being compared against.

A by-product worth stating explicitly: the monotone convergence establishes that the oscillations trailing a sharp second-sound front in spectral discretizations of this problem are a Gibbs artefact, not a physical or numerical instability. This is often asserted; here it is measured.

[Figure 2: figures_validation/ls_wave_validation_R1.png. Table: the mesh-convergence and time-error data, available as thesis_chapter/ls_wave_mesh_convergence_R1.csv and ls_wave_time_errors_R1.csv.]

## 4. Spatial discretizations compared

Five discretizations are run on the transient-conduction problem with the exact Bessel-series solution, using an identical Newmark march so that the temporal error is common to all of them.

| method | nodes to reach the temporal floor |
| --- | --- |
| Differential quadrature, Chebyshev grid | ≈ 9–11 |
| Differential quadrature, uniform grid | ≈ 11 |
| Finite elements, quadratic (3-node) | ≈ 21 |
| Finite elements, linear (2-node) | ≈ 161 |
| Finite differences, second order | ≈ 161 |

The linear-FEM and FDM curves lie on the slope −2 reference line, confirming second-order convergence; quadratic FEM gains roughly one order of magnitude per refinement step; the differential quadrature error falls quasi-exponentially until it meets the temporal floor. At equal accuracy the DQM system is therefore about fifteen times smaller per direction, and that ratio is squared in the (r,z) plane — the quantitative justification for the layerwise DQM choice, and consistent with the mesh study in which a handful of radial points already delivered three-digit accuracy.

The finite-element comparison uses a standard Bubnov–Galerkin weighted-residual formulation with the cylindrical weight r in the weak form, C⁰ Lagrange elements, consistent capacitance matrices, natural Robin boundary conditions and essential Dirichlet conditions. Because the transient conduction operator is self-adjoint — there is no convection term — Bubnov–Galerkin is optimal in the best-approximation sense and no Petrov–Galerkin upwinding is required; this is worth stating because referees in this area routinely ask.

### 4.1 Differential quadrature has a window, not a curve

The most practically useful finding of this section is a negative one. The DQM error does not keep falling. On the Chebyshev grid it flattens after N≈11, drifts slightly upward through N≈21–121, and the solution then fails outright at N≈161. On the uniform grid the degradation is violent: the error is competitive at N=11–31 and then diverges by tens of orders of magnitude by N=41.

The mechanism is the well-known ill-conditioning of the differential quadrature weighting matrices, whose condition number grows rapidly with the number of nodes; on a uniform grid it grows faster still. The practical consequence is that DQM must be used inside a window whose upper edge should be located before production runs, and that the usual habit of “refine until the answer stops changing” is unsafe here — refinement past the window degrades the answer silently before it fails loudly. Since the method’s whole advantage is that it needs few nodes, this is not a serious restriction, but it does need to be stated.

[Figure 3: results_extensions/figures/spatial_convergence — log-log error against N for all five methods with the slope −2 reference line.]

### 4.2 The binding direction

A companion study varying Nr, Nz and NL independently about a fixed anchor shows that the axial direction is the binding one: Nz=5 still carries a 9.4 % error in the hoop-stress profile, crossing below 1 % only by Nz=7–9, while Nr and NL are already within a few tenths of a percent at their coarsest tested values. Effort in this class of problem is conventionally spent on radial refinement; this result says it is better spent axially.

### 4.3 A caution on end conditions

Not all mechanically admissible end conditions are numerically admissible in this discretization. With simply supported or clamped ends the solution is stable. With free or roller ends it diverges — isolation tests confirm the cause is spurious mechanical end modes arising from the absence of kinematic anchoring of the displacement field at the axial boundaries, and that the divergence is independent of the Newmark parameters and of the thermomechanical coupling. The practical rule is that the u-field requires kinematic anchoring at the axial ends in a layerwise DQM formulation of this problem; one-dimensional radial problems avoid the issue entirely.

## 5. Time integrators compared

Six schemes on the identical spatial system, measured against the exact solution.

| method | max error (K) | CPU (s) |
| --- | --- | --- |
| Newmark (δ = ½, β = ¼) | 0.0032 | 0.33 |
| Wilson-θ (θ = 1.4) | 0.0164 | 0.19 |
| Houbolt | 0.0025 | 0.24 |
| HHT-α (α = −0.1) | 0.0033 | 0.29 |
| Adaptive stiff solver (NDF/BDF) | 0.00026 | 2.57 |
| Laplace–Durbin (thermal subsystem only) | 0.137 | 193 |

Newmark, Houbolt and HHT-α are of practically equal accuracy at equal step. Wilson-θ is the cheapest but accepts a roughly five-fold error for its additional numerical dissipation. The adaptive stiff solver is an order of magnitude more accurate but roughly eight times more expensive, and it is worth noting what it required: because the semi-discrete system is a differential-algebraic one, the constraint rows had to be eliminated by static condensation and the remaining second-order system recast in first-order state-space form before the solver could be applied at all, with an analytic Jacobian supplied to keep the cost down. The default formula in that solver is the modified NDF family rather than classical BDF, which is worth stating precisely since the distinction is routinely blurred in the applied literature.

Newmark’s combination of accuracy, cost and implementation simplicity justifies it as the production choice — a conclusion that is unsurprising but that, as far as the authors are aware, has not previously been measured for this class of problem.

## 6. Why the Laplace route fails on the coupled system

The Laplace-transform method with numerical inversion is the slowest entry in the table, but its real interest is that its failure is structural rather than one of efficiency.

The obstruction. Numerical inversion evaluates the transformed solution along a contour in the complex plane. For the thermal subsystem alone the conduction poles lie on the negative real axis, comfortably away from any sensible contour, and the inversion is well behaved — merely expensive. For the fully coupled system the operator additionally carries the elastic poles, and those are undamped: they lie on the imaginary axis, which is exactly where the Durbin contour samples. Every contour evaluation is then a near-singular solve, and the alternating sum that reconstructs the time-domain solution is poisoned by the resulting cancellation. No refinement of the inversion parameters repairs this, because the difficulty is in the location of the spectrum, not in the quadrature.

This provides the explanation, usually left implicit, for why transform methods in the generalized-thermoelasticity literature are applied to thermal subproblems and not to fully coupled ones.

Two measured failures support the diagnosis. A fixed-Talbot inversion applied to the wave problem produced errors that grew with the number of nodes — 3.2 × 10¹, 8.6 × 10², 4.2 × 10⁸ K — because the transformed field behaves as a pure delay, ∼e−sΔr/C/s, and Talbot’s node radius grows as e2M/5, amplifying exactly the cancellation that a pure delay produces. This is why the modal route of §3 was needed for the benchmark.

An arithmetic caution worth recording. In implementing the Durbin inversion, the half-period node spacing sk=a+ikπ/T with T=tend and prefactor (1/T)eat12F(a)+∑, with a=7/(2T), is required. Using full-period spacing instead yields a result that is uniformly and exactly twice the correct one — a discrepancy that is easy to mistake for a modelling error and that was isolated here by testing against an analytically invertible transform. Early times shorter than Tper/(2N) are unresolvable in any case.

## 7. Conclusions

- Differential quadrature reaches the temporal error floor at roughly 9–11 nodes per direction against approximately 161 for linear finite elements and second-order finite differences and 21 for quadratic finite elements — about fifteen times fewer per direction, squared in the plane.

- Differential quadrature has a usable window rather than a monotone convergence curve: beyond it the error degrades and then the solution fails, catastrophically on a uniform grid. Refining until the answer stops changing is not a safe procedure here.

- The axial direction is the binding one for mesh convergence in this geometry, contrary to where refinement effort is usually directed.

- Free and roller end conditions excite spurious mechanical end modes in this formulation; the displacement field requires kinematic anchoring at the axial ends.

- Newmark, Houbolt and HHT-α are of equal accuracy at equal step; Wilson-θ trades a five-fold error for its dissipation; adaptive NDF/BDF integration gains a decimal digit at eight times the cost and requires condensation to a first-order form first.

- The Laplace route fails structurally on the fully coupled operator because the undamped elastic poles lie on the inversion contour — the reason transform methods in this field are confined to thermal subsystems.

- An exact Fourier–Bessel benchmark for the Lord–Shulman wave in a hollow cylinder is provided and independently verified, and is offered as a reusable reference for second-sound codes.

## References

- Bellman R, Casti J. Differential quadrature and long-term integration. J Math Anal Appl 1971;34(2):235–8.

- Shu C. Differential Quadrature and Its Application in Engineering. Springer; 2000.

- Bert CW, Malik M. Differential quadrature method in computational mechanics: a review. Appl Mech Rev 1996;49(1):1–28.

- Newmark NM. A method of computation for structural dynamics. J Eng Mech Div ASCE 1959;85(3):67–94.

- Hilber HM, Hughes TJR, Taylor RL. Improved numerical dissipation for time integration algorithms in structural dynamics. Earthq Eng Struct Dyn 1977;5(3):283–92.

- Houbolt JC. A recurrence matrix solution for the dynamic response of elastic aircraft. J Aeronaut Sci 1950;17(9):540–50.

- Wilson EL, Farhoomand I, Bathe KJ. Nonlinear dynamic analysis of complex structures. Earthq Eng Struct Dyn 1972;1(3):241–52.

- Durbin F. Numerical inversion of Laplace transforms: an efficient improvement to Dubner and Abate’s method. Comput J 1974;17(4):371–6.

- Talbot A. The accurate numerical inversion of Laplace transforms. J Inst Math Appl 1979;23(1):97–120.

- Shampine LF, Reichelt MW. The MATLAB ODE suite. SIAM J Sci Comput 1997;18(1):1–22.

- Lord HW, Shulman Y. A generalized dynamical theory of thermoelasticity. J Mech Phys Solids 1967;15(5):299–309.

- Bagri A, Eslami MR. A unified generalized thermoelasticity; solution for cylinders and spheres. Int J Mech Sci 2007;49(12):1325–35.

- Reddy JN. An Introduction to the Finite Element Method. McGraw-Hill.

- [Paper 1 of this series — cite as submitted / in press.]

## Submission checklist

- The DQM-breakdown subsection (§4.1) is the most likely to draw referee pushback — have the condition-number data ready as a response even if it is not in the paper

- Publish the benchmark data as supplementary material; it is much of the paper’s reuse value

- Confirm the exact-solution derivation is written out in full in an appendix

- Verify no figure overlaps Papers 1 and 2

- Author order and thesis footnote