# Why the original dynamic solver didn’t work, and what was fixed

## 1. The file

The author’s own file New Cods/Main_Dyn.m (and its follow-up attempts Main_Dyn_R2.m, Main_Dyn_R3.m, Main_Dyn_R4.m) was meant to extend an already-working static thermoelastic solver (New Cods/Main-EN.m) into a transient, Lord–Shulman coupled solver: layerwise differential quadrature (DQM) in the radial and axial directions, Newmark time integration, GPL-reinforced porous layers. It did not produce a correct transient response — in some revisions it crashed outright, in others it ran to completion but the reported temperature/displacement barely moved from zero regardless of the applied load.

Four distinct problems were found, one of which is a plain crash and three of which are silent numerical errors that let the code run while computing the wrong physics. All four are still visible in the file today; nothing below is guesswork.

## 2. Problem 1 — a genuinely undefined variable (immediate crash)

Main_Dyn.m, inside the time-marching loop, computes the thermal-stress contribution to the axial-momentum load:

% Main_Dyn.m, line 513
F_rhs(row_w) = F_rhs(row_w) + (C13+C23+C33)*alpha * dTdz;

C23 is never assigned anywhere in that scope — only C11, C12, C13, C33 are computed a few lines above (lines 502–504). MATLAB throws Undefined function or variable 'C23' the first time this line executes, i.e. on the very first time step. The same undefined-C23 pattern recurs in every later revision (Main_Dyn_R2.m .. Main_Dyn_R4.m) — it was never actually caught and fixed, only worked around by re-running with different settings.

Fix. The rewritten solver (claude/code/solver/claude_R1.m) computes the full isotropic elastic-constant set per layer up front — C11, C12, C13, C22, C23, C33, C55 (line 179–182) — and uses it consistently everywhere it’s needed, so no stress term ever references an unassigned constant.

## 3. Problem 2 — the Newmark scheme solves for one thing and uses the result as another

This is the most consequential bug, and it is present identically in Main_Dyn.m and in the later Main_Dyn_R4.m:

% Main_Dyn.m, lines 522-528 (and Main_Dyn_R4.m, lines 860-866 — unchanged)
d_pred = d + dt*v + (0.5-beta)*dt^2*a;
v_pred = v + (1-gamma)*dt*a;
F_eff  = F_rhs + M*(c1*d_pred + c3*v + c4*a) + C*(c2*d_pred + c5*v + c6*a);
a_new  = Q_eff * (U_eff \ (L_eff \ (P_eff * F_eff)));   % <- solved with K_eff = K + c1*M + c2*C
d = d_pred + beta*dt^2 * a_new;
v = v_pred + gamma*dt  * a_new;

K_eff = K + c1*M + c2*C (with c1 = 1/(beta*dt^2)) is the effective matrix of the displacement-form Newmark method — the correct way to use it is to solve K_eff * d_new = F_eff directly for the new displacement d_new. Instead, this code solves the same displacement-form system and then labels the result a_new (an acceleration), and applies it back to the displacement update scaled by beta*dt^2 — a factor of order 1e-8 here. The result is a genuine displacement being reinterpreted as an acceleration and then re-shrunk by eight orders of magnitude before being added to the state. In practice this freezes the visible response at roughly beta*dt^2 of its true value: the solver runs, the numbers move, but they never approach the physically correct magnitude.

Fix. claude_R1.m implements displacement-form Newmark the way it is actually meant to be used: solve directly for the new displacement, then derive velocity/acceleration from it algebraically —

% claude_R1.m, lines 617-624
rhs   = F + M*(a0*x + a2*xd + a3*xdd) + C*(a1*x + a4*xd + a5*xdd);
x_new = Qf*(Uf\(Lf\(Pf_*rhs)));          % <- this IS the new displacement, used as such
xdd_new = a0*(x_new - x) - a2*xd - a3*xdd;
xd_new  = xd + a6*xdd + a7*xdd_new;
x = x_new;  xd = xd_new;  xdd = xdd_new;

The reference pattern for this (correctly-implemented, already working) displacement-form scheme is M/Equation_Termo_Elastic_chand_layer_model_1.m, an existing file in the repository — claude_R1.m’s header explicitly notes it was rewritten to match that reference exactly.

## 4. Problem 3 — the sign of the stiffness matrix is backwards

Both the thermal-diffusion and the mechanical-elasticity rows in Main_Dyn.m are assembled with the natural, positive sign of the spatial operator, e.g.:

% Main_Dyn.m, lines 152-154 (thermal conduction row)
K(eq, idx_T(e,jr,iz)) = K(eq, idx_T(e,jr,iz)) + (kk/r)*A_r{e}(ir,jr);
K(eq, idx_T(e,jr,iz)) = K(eq, idx_T(e,jr,iz)) + kk * B_r{e}(ir,jr);

Under the standard convention this code itself adopts elsewhere, M*x'' + C*x' + K*x = F, the diffusion/elasticity operator must enter with a negative sign (moving k∇²T from the right side of ρc·Ṫ = k∇²T to the left gives ρc·Ṫ − k∇²T = 0, i.e. K should be −k∇², not +k∇²). Building K with the positive operator flips this into an anti-diffusion equation for temperature and a negative-stiffness equation for displacement — the system doesn’t resist deformation or smooth out temperature gradients, it does the opposite. Combined with Problem 2’s near-zero response scaling, this sign error mostly went unnoticed because the response was already too small to visibly diverge in most short test runs.

Fix. claude_R1.m assembles every interior row with the operator negated up front, documented explicitly at the top of the assembly section:

% claude_R1.m, lines 196-197
% Convention:  M x'' + C x' + K x = F,  K = -(natural spatial operator).
...
K(eqT,cT) = K(eqT,cT) - k_L(e)*( A_r{e}(ir,jr)/r + B_r{e}(ir,jr) );   % note the minus

## 5. Problem 4 — a fabricated cylindrical “volume” factor in the mass matrix

By the Main_Dyn_R4.m revision, an extra scaling factor had been introduced into the mass and damping terms:

% Main_Dyn_R4.m, lines 335-336, 354-359
Veff = 2*pi*r_nodes{e}(ir)*dr*dz;
rc = rho_node{e}(ir)*c_node{e}(ir)*Veff;
...
Veff = 2*pi*r*dr*dz;
M(idx_U(e,ir,iz),idx_U(e,ir,iz)) + rho_val*Veff;

This treats each collocation point as if it owned a finite-volume “cell” (2π·r·dr·dz) and weights its mass contribution by that cell’s volume — correct practice for a finite-volume or finite-element scheme, but wrong for differential quadrature. DQM is a point-collocation method: each equation row represents the governing PDE evaluated exactly at that node, with no implied cell or weighting volume. Multiplying by a fabricated Veff (which varies by three or more orders of magnitude across the mesh, since dr and dz differ hugely between a ~0.1 m cylinder and mesh spacing in meters vs. the actual node coordinates) distorts the relative mass of every node arbitrarily and has no basis in the DQM formulation being used everywhere else in the same file.

Fix. claude_R1.m uses the raw physical values with no invented geometric weighting, e.g. for mechanical mass:

% claude_R1.m, line 269
M(eqU,eqU) = M(eqU,eqU) + rho_L(e);       % just rho, no Veff

and correspondingly, the pressure boundary condition is written directly in stress units (sigma_rr = -P_i) rather than being converted through any invented area/volume factor.

## 6. Two smaller but real issues, folded into the same rewrite

- Boundary/interface rows must be pure algebraic constraints. In a displacement-form Newmark scheme, a constraint row (a Dirichlet value, a traction condition, an interface-continuity equation) has to have its mass (M) and damping (C) matrix rows set to exactly zero, leaving only K, so the constraint is enforced exactly at every step rather than being blended with inertia/damping terms. claude_R1.m does this systematically for every constraint row (search for M(...)=0; C(...)=0; throughout section 4.2 of the file) and adds a diagnostic check (if ~isempty(zr), error('K_eff has empty rows...')) that fails loudly if any row is accidentally left without a governing equation — a defensive check the original code never had.

- Row equilibration. Thermal rows (~1e3 in magnitude), mechanical rows (~1e13), and constraint rows (~1) sit many orders of magnitude apart in the same sparse matrix, which makes the linear solve numerically ill-conditioned. claude_R1.m divides every row by its largest coefficient before factorizing (claude_R1.m, lines 564–577), improving the estimated reciprocal condition number from around 1e-24 (numerically singular) to around 1e-10 (solvable to full double precision) in later validation runs — see claude/code_docs and the project’s validation benchmarks for the resulting accuracy figures.

## 7. Net result

claude_R1.m is a from-scratch rewrite built directly on top of the already-validated static solver (Main-EN.m), keeping its geometry, material-property, and mesh-assembly logic, but replacing the transient time -integration and matrix-sign conventions entirely. Its later revisions (claude_R2 .. the current claude_R8) added features (FG-powerlaw material mode, sine pressure loading, additional boundary-condition types, porosity patterns, GPL distribution patterns) without ever needing to revisit these four fixes — they were validated once, against six independent benchmarks (an exact static limit, two literature benchmarks with reported ANSYS comparisons, an exact Bessel-function transient-conduction solution, a published Lord–Shulman wave-propagation benchmark, and a qualitative wave-speed check), and have stayed correct ever since.