# Appendix A. The Newmark time-integration method and the comparison methods

This appendix describes the Newmark time-integration scheme used for all transient solutions of this thesis, and briefly the alternative time-integration and spatial-discretization methods used in the method-comparison studies of Chapter 4 (Sections 4-18 to 4-19).

## A.1 The semi-discrete system

After the layerwise differential-quadrature (DQM) discretization in the (r,z) plane described in Chapter 3, the coupled Lord-Shulman thermoelastic problem is reduced to a system of ordinary differential equations in time for the vector of nodal unknowns

x=θuw,

where θ, u and w collect the nodal temperature increment θ=T−T∞ and the radial and axial displacements at all (r,z) grid points of all layers. The semi-discrete system is written in the unified second-order form

M x+C x+K x=F(t).

The three matrices are assembled directly from the governing equations:

- Mechanical rows (u,w): the momentum equations ρu−(Lu)r+β ∂rθ=0 are second order in time, so they contribute ρ to M, nothing to C (elastodynamics is undamped), and the elastic operator together with the thermoelastic coupling β ∂θ to K.

- Thermal rows (θ) under Lord-Shulman: the energy equation ρc(θ+τ0θ)+βT0(e+τ0e)−∇​⋅(k∇θ)=0 contributes ρc τ0 (and the coupling βT0τ0 e) to M, ρc (and βT0 e) to C, and the conduction operator −∇​⋅(k∇ ⋅) to K.

Here e=∇​⋅u is the dilatation, τ0 the relaxation time, β=α(3λ+2μ) the thermal modulus of the homogenized material, and T0 the reference temperature. The relaxation term τ0θ is exactly what makes the thermal rows carry a mass term and turns the parabolic Fourier system into the hyperbolic (wave-type) Lord-Shulman system. Setting τ0=0 recovers the classical coupled Fourier model, in which the thermal rows have no mass term.

## A.2 The Newmark scheme

The system is advanced in time with the Newmark β family. Given the state (xn,xn,xn) at time tn, the state at tn+1=tn+Δt is defined by the two Newmark approximations

xn+1=xn+Δt xn+Δt22(1−2β) xn+2β xn+1,

xn+1=xn+Δt(1−γ) xn+γ xn+1.

Substituting these into the equation of motion at tn+1 and eliminating xn+1 and xn+1 gives a single linear system for xn+1 (the displacement form of the Newmark method):

Keff xn+1=Fn+1+Ma0xn+a2xn+a3xn+Ca1xn+a4xn+a5xn,

with the effective stiffness matrix

Keff=K+a0M+a1C,

and the integration constants

a0=1β Δt2, a1=γβ Δt, a2=1β Δt, a3=12β−1,

a4=γβ−1, a5=Δt2​γβ−2, a6=Δt (1−γ), a7=Δt γ.

Once xn+1 is obtained, the acceleration and velocity are updated by

xn+1=a0xn+1−xn−a2xn−a3xn,

xn+1=xn+a6 xn+a7 xn+1.

### Choice of parameters

Throughout this thesis the parameters are set to γ=12 and β=14, which correspond to the constant-average-acceleration (trapezoidal) rule. This choice is:

- unconditionally stable for the linear system, so the time step Δt is limited only by accuracy, not by stability;

- second-order accurate in time; and

- free of algorithmic (numerical) damping, because γ=12.

The absence of numerical damping is deliberate: the mild oscillations visible behind the sharp thermal wave fronts in the results of Chapter 4 are the genuine Gibbs oscillations of the spectral (DQM) spatial discretization, and are reported without artificial smoothing so that the physical wave response is not masked. A value γ>12 would add numerical dissipation that damps these oscillations but also artificially attenuates the physical thermal wave; it is therefore avoided here. (The symbol γ used for the percolation exponent of the conductivity model in Chapter 3 is unrelated to the Newmark γ; to avoid confusion the Newmark parameter is written δ in the main text and γ only within this appendix.)

### Computational remarks

Because Keff is constant for a fixed time step (the problem is linear), it is factorized once by an LU decomposition before the time loop, and each step reuses the factors in forward/backward substitution; only the right-hand side is rebuilt each step. Before factorization every equation row is equilibrated — divided by its largest coefficient — so that the thermal rows (of magnitude ∼103), the mechanical rows (∼1013) and the constraint rows (∼1) attain comparable magnitudes; this row scaling reduces the condition number of Keff by many orders of magnitude and is essential for an accurate solution of the strongly heterogeneous coupled system. The reference solution uses Δt small enough that the time-discretization error lies below the spatial error floor, as verified by the convergence study of Section 4-3.

## A.3 The comparison methods

To position the Newmark scheme and the DQM spatial discretization, several alternative methods were implemented on identical benchmark problems; their results are reported in Chapter 4 and summarized here.

### A.3.1 Alternative time integrators (Section 4-18, Table 4-3)

Five alternative time integrators were run on the same spatial system and the same transient-conduction benchmark (with a known exact solution):

- Wilson-θ (θ=1.4): an implicit scheme that evaluates the equilibrium at tn+θΔt; unconditionally stable but with extra numerical dissipation, giving a roughly fivefold larger error than Newmark at equal step.

- Houbolt: a backward multi-step scheme built from a cubic backward difference; comparable accuracy to Newmark but with strong high-frequency damping.

- HHT-α (α=−0.1): the Hilber-Hughes-Taylor generalization of Newmark that introduces controlled high-frequency damping while retaining second-order accuracy; practically as accurate as Newmark here.

- ode15s: MATLAB’s adaptive, variable-order stiff (BDF) solver, used as an independent reference; about ten times more accurate but about eight times more expensive than Newmark.

- Laplace-Durbin: a Laplace-transform-in-time method with Durbin’s numerical inversion, applicable only to the thermal subsystem. It is three orders of magnitude slower and, importantly, cannot be applied to the full coupled system, because the undamped elastic poles of the coupled operator lie exactly on the Bromwich inversion contour — which is precisely why transform methods appear in the literature only for uncoupled thermal problems.

Newmark, Houbolt and HHT-α are practically equally accurate at equal step; Newmark offers the best overall accuracy-to-cost ratio and was therefore adopted for all production runs.

### A.3.2 Alternative spatial discretizations (Section 4-19, Figure 4-2)

Four spatial discretizations were compared on the transient-conduction benchmark with the same Newmark time march:

- Layerwise differential quadrature (DQM) with Chebyshev-Gauss-Lobatto and with uniform grids: the temperature/displacement derivatives at each node are written as weighted sums of all nodal values along a line, giving a spectral (quasi-exponential) convergence rate.

- Second-order finite differences (FDM): standard central differences.

- Finite elements (FEM), Galerkin formulation with consistent capacitance and conductance matrices, using linear (2-node) and quadratic (3-node) Lagrange elements. Because the transient-conduction operator is self-adjoint (no convection), the standard Bubnov-Galerkin method (test = trial functions) is optimal and Petrov-Galerkin weighting is not needed.

The number of radial points required to reach the time-step error floor is N≈9-11 for DQM, N≈21 for quadratic FEM, and N≈161 for linear FEM and second-order FDM. At equal accuracy the DQM equation system is therefore about fifteen times smaller per direction — squared in the (r,z) plane — which is the quantitative justification for the layerwise differential-quadrature choice adopted throughout this thesis.