# Abstract

Thermoelastic Analysis of Porous Graphene-Platelet-Reinforced Cylinders under Thermo-Mechanical Loading Using the Lord-Shulman Theory

This thesis investigates the thermoelastic analysis of porous cylinders reinforced with graphene platelets under thermo-mechanical loading, using Lord-Shulman theory. The hollow cylinders studied consist of layers with properties that vary through the thickness, arising from changes in the graphene platelet volume fraction and the porosity percentage in each layer.

The temperature distribution, stresses, strains, and displacements in these hollow cylinders are examined. For this purpose, the Lord-Shulman equations of motion and energy conservation are used to derive the governing equations, boundary conditions, and compatibility conditions. The efficient differential quadrature method is then used to discretize the equations of motion, boundary conditions, and compatibility conditions in space, and the Newmark method is used in time.

The elastic properties of the layers are calculated using the Halpin-Tsai theory and the rule of mixtures. The open-cell theory is also used to account for property changes due to porosity.

Because the Lord-Shulman theory limits the propagation speed of the thermal wave, the accuracy of the temperature calculation increases, and the temperature history shows a “peak-trough” pattern caused by thermal wave reflection. Placing conductive graphene platelets at the hot inner surface together with insulating porosity at the cold outer surface acts synergistically and markedly reduces the outer-surface temperature. Under thermal shock loading, the classical Fourier theory underestimates the peak temperature by about 2.5-fold, which is a strong argument in favor of the generalized theory. Methodologically, the combination of differential quadrature and Newmark integration achieves the desired accuracy at low computational cost.

This work is the first fully coupled Lord-Shulman analysis of a multilayer porous graphene-reinforced cylinder, and its principal contribution is the complete interaction map of all twenty-five pattern pairings: the alignment or opposition of the two gradings, not either pattern alone, governs the thermal behaviour, so they cannot be chosen independently. An outer-concentrated porosity forms a universal thermal barrier on every graphene background and turns the inner-surface hoop stress compressive; screening on temperature alone is nevertheless unsafe, since a thermally mild pairing can carry the largest hoop stress of the matrix. A small graphene fraction switches the wall from a poor to a good conductor and further addition mainly raises the stresses, while the platelet geometry is only a second-order lever at this fraction. A four-theory map (Fourier, Lord-Shulman, dual-phase-lag and Green-Naghdi) shows that Green-Naghdi type III differs qualitatively for want of a dissipative mechanism, and full coupling damps the thermal wave, so under impulsive loading both the classical theory and uncoupled models are non-conservative. Verification rests on five independent tests, among which the derivation of an exact Fourier-Bessel second-sound solution is itself a reusable contribution.

Keywords: thermoelastic analysis, porous cylinder, graphene platelets, thermo-mechanical loading, Lord-Shulman theory.