# Exact Bessel-Series Solution of Transient Heat Conduction in a Hollow Cylinder — Full Equations and Step-by-Step Derivation

Validation test 3 (Section 4-4-3) — author reference note

# 0. Overview

Problem. Transient Fourier heat conduction in a homogeneous hollow cylinder, in the radial direction only. The inner surface temperature rises as an exponential ramp and the outer surface exchanges heat by convection.

Idea.

- The inhomogeneous boundary condition is removed with a unit steady profile ψ(r).

- What remains is a problem that is homogeneous in space. It is expanded in Bessel eigenfunctions (combinations of J0 and Y0).

- The amplitude of each mode obeys a first-order ordinary differential equation in time, which is solved exactly.

No spatial or temporal discretisation is involved. The only approximation is truncating the series to N modes.

Final result. With β=1/t0 and T=T∞+θ:

θ(r,t)=θ01−e−βt ψ(r) − θ0n=1Ncn Rn(r) βe−βt−e−κλn2tκλn2−β

# 1. Problem statement

## 1.1 Material, geometry and loading (test 4-4-3)

| Quantity | Value |
| --- | --- |
| Conductivity | k=50 W/(m K)​ |
| Density | ρ=8000 kg/m3​ |
| Specific heat | c=500 J/(kg K)​ |
| Thermal diffusivity | κ=k/(ρc)=1.25×10−5 m2/s​ |
| Radii | Ri=0.08 m, Ro=0.1 m |
| Outer convection | h=2000 W/(m2 K)​ |
| Biot number | Bi=h(Ro−Ri)/k=0.8​ |
| Ramp amplitude and time constant | θ0=200 K, t0=5 s |
| Reference temperature | T∞=300 K​ |

## 1.2 Why the problem is one-dimensional

Both ends of the cylinder are insulated, the material is homogeneous, and the inner and outer boundary conditions do not depend on z. Hence the temperature field depends on (r,t) only.

## 1.3 Governing equation and conditions

With θ=T−T∞:

∂θ∂t=κ∂2θ∂r2+1r∂θ∂r,  Ri<r<Ro, t>0

θ(Ri,t)=f(t)≡θ01−e−βt,  k θr(Ro,t)+h θ(Ro,t)=0,  θ(r,0)=0.

The outer condition is a Robin (convective) condition: the conductive flux reaching the surface equals the convective flux leaving it, −kθr=hθ.

# 2. Step 1 — Removing the inhomogeneous boundary condition with the unit steady profile

Separation of variables needs homogeneous boundary conditions, but the inner condition f(t) is inhomogeneous. Split the solution into two parts:

θ(r,t)=f(t) ψ(r)+v(r,t),

where ψ is the steady profile with unit inner temperature:

∇2ψ=0,  ψ(Ri)=1,  k ψ′(Ro)+h ψ(Ro)=0.

The general solution of ψ″+ψ′/r=0 is ψ=a+blnr. The inner condition gives a=1−blnRi, and the outer Robin condition gives

k bRo+h1+blnRoRi=0 ⇒  b=−hk/Ro+hln(Ro/Ri),  a=1−blnRi

Numbers. b=−2.11352, a=−4.33819, ψ(Ro)=a+blnRo=0.52838. In the steady state the outer surface therefore receives 52.8 % of the inner amplitude, i.e. To∞=300+200×0.52838=405.68 K.

Two properties of ψ used later.

- r ψ′(r)=b is constant, hence (rψ′)′=0.

- From the Robin condition, b=Ro ψ′(Ro)=−Ro (h/k) ψ(Ro).

# 3. Step 2 — The remaining problem for v

Because ∇2(fψ)=0, only the time-derivative term of fψ survives:

vt=κ∇2v−f′(t) ψ(r),  v(Ri,t)=0,  k vr+h v=0  (r=Ro),  v(r,0)=0.

The initial condition of v is zero because f(0)=0. This compatibility between the boundary and initial conditions is the reason for using an exponential ramp rather than a step. The source term is −f′(t)ψ(r)=−θ0β e−βtψ(r), a pure exponential in time.

# 4. Step 3 — Eigenfunctions: Bessel’s equation

Expand the spatial part of v in the eigenfunctions of ∇2R+λ2R=0:

R″+1rR′+λ2R=0 ⇔ (rR′)′+λ2 r R=0.

With x=λr this is Bessel’s equation of order zero, with general solution A J0(λr)+B Y0(λr). Since the domain excludes r=0, both functions are admissible. The inner Dirichlet condition R(Ri)=0 fixes the ratio A:B:

Rn(r)=J0(λnr) Y0(λnRi)−Y0(λnr) J0(λnRi)

Using ddrJ0(λr)=−λJ1(λr) and the same rule for Y0:

Rn′(r)=−λnJ1(λnr) Y0(λnRi)−Y1(λnr) J0(λnRi).

# 5. Step 4 — The Robin condition gives the eigenvalue equation

Substituting into k R′(Ro)+h R(Ro)=0:

G(λ)=−kλJ1(λRo)Y0(λRi)−Y1(λRo)J0(λRi)+hJ0(λRo)Y0(λRi)−Y0(λRo)J0(λRi)=0

The positive roots λ1<λ2<⋯ are the eigenvalues. For k,h>0 all λ2 are real and positive; this follows from the self-adjointness of the problem and from the positivity of ∫rR′​2 dr+hkRoR(Ro)2. The roots have no closed form and are found numerically.

Structure of the roots. For large λ the Bessel functions behave like sines, and

Rn≈2πλrRi sinλ(Ri−r),

so the root spacing is approximately π/(Ro−Ri)=157 m−1. Computed values:

| n​ | 1 | 2 | 3 | 4 | 10 | 60 |
| --- | --- | --- | --- | --- | --- | --- |
| λn (m−1)​ | 95.867 | 242.71 | 397.06 | 552.91 | 1493.4 | 9346.4 |
| 1/(κλn2) (s)​ | 8.70 | 1.36 | 0.51 | — | — | — |

# 6. Step 5 — Orthogonality with weight r

For two eigenfunctions Rm,Rn, the relation (rR′)′=−λ2rR gives

(λm2−λn2)RiRor RmRn dr=[rRmRn′−RnRm′]RiRo.

At Ri both functions vanish. At Ro the Robin condition R′=−(h/k)R gives RmRn′−RnRm′=−(h/k)(RmRn−RnRm)=0. Therefore, for λm≠λn,

RiRoRm(r) Rn(r) r dr=0.

(In MATLAB the normalised orthogonality for n=1,2 came out as 2.3×10−16.) The norm is

Nn=RiRoRn2 r dr=Ro22Rn(Ro)2+Rn′(Ro)2λn2−Ri22 Rn′(Ri)2λn2,

which follows from Lommel’s identity ∫r Z0(λr)2dr=r22Z02+Z12 for any order-zero cylinder function Z0, together with Rn(Ri)=0.

# 7. Step 6 — Expansion coefficients cn

Expand ψ in the basis {Rn}: ψ=n​cnRn. By orthogonality,

cn=RiRoψ Rn r drRiRoRn2 r dr.

The code evaluates both integrals numerically. The numerator, however, has a closed form (derived here and checked numerically).

Starting from (rRn′)′=−λn2rRn, integrating by parts twice and using (rψ′)′=0:

λn2RiRor ψRn dr=−[rψRn′]RiRo+[rψ′Rn]RiRo=−ψ(Ro)RoRn′(Ro)+ψ(Ri)RiRn′(Ri)+b Rn(Ro).

With the Robin condition Rn′(Ro)=−(h/k)Rn(Ro) and property (b) of Section 2 (b=−Ro(h/k)ψ(Ro)), the first and third terms cancel exactly, leaving only RiRn′(Ri). The Wronskian identity J1(x)Y0(x)−J0(x)Y1(x)=2πx gives Rn′(Ri)=−λn⋅2πλnRi=−2πRi. Hence

RiRoψ Rn r dr=−2πλn2,  cn=−2πλn2 Nn

Numerical check (λ1,…,λ4). The closed form and the numerical integral agree to all printed digits (for n=1 both give −6.926908×10−5), and Nn from the integral and from Lommel’s formula agree (for n=1: 6.413852×10−6). The coefficients are

c1,…,c5=−10.800, −12.218, −12.432, −12.497, −12.524.

Why cn does not decay with n. For large λ, Nn≈2(Ro−Ri)π2λn2Ri, and therefore

cn → −πRiRo−Ri=−12.566.

The reason is that ψ(Ri)=1 while every Rn(Ri)=0, so the expansion of ψ converges non-uniformly (Gibbs-like) at the inner surface. In the final solution this is compensated by the time factor 1/λn2 (next step). It also helps that ψ is multiplied by f(t) outside the series and therefore satisfies the inner condition exactly; the series part v vanishes at Ri by construction.

# 8. Step 7 — Time equation of each mode (Duhamel)

Insert v=n​vn(t)Rn(r) into the equation of Step 2. Since ∇2Rn=−λn2Rn and ψ=∑cnRn, the coefficient of each Rn must vanish separately:

vn+κλn2 vn=−cn f′(t)=−cn θ0β e−βt,  vn(0)=0.

This is a linear first-order equation. With the integrating factor eκλn2t (Duhamel integral):

vn(t)=−cnθ0β0te−βs e−κλn2(t−s) ds=−cn θ0 βe−βt−e−κλn2tκλn2−β.

Technical remark. If κλn2=β the denominator vanishes, but the singularity is removable (the limit is t e−βt). For β=0.2 s−1 this happens at λ=β/κ=126.5 m−1, which lies between λ1=95.87 and λ2=242.7 and is not an eigenvalue. So it never occurs in this test; it may come close if t0 is changed.

# 9. Step 8 — Final solution

θ(r,t)=θ01−e−βtψ(r) − θ0n=1∞cn Rn(r) βe−βt−e−κλn2tκλn2−β

with ψ=a+blnr, Rn from Step 3, λn the roots of G=0, and cn=−2/(πλn2Nn).

Limiting behaviour (for physical insight).

- t→∞: all exponentials decay and θ→θ0ψ(r), the steady logarithmic profile.

- t→0: fψ→0 and the series vanishes, so the initial condition holds (check: θ(0.09,0)=0 exactly).

- Higher modes die quickly (e−κλn2t with λn∝n). At t=40 s only the first mode remains (time constant 8.7 s, i.e. e−4.6=1 %).

- Convergence of the series: term n behaves like cn |Rn| β/(κλn2) with |Rn|≲2/(πλnrRi), that is O(λn−3)∼O(n−3), so the series converges absolutely.

# 10. Step 9 — Numerical evaluation

- Root finding. G(λ) is sampled on the grid λ∈[1, 4×104] with 400 000 points (step ≈0.1); sign changes are located and each bracket is refined with a root finder (fzero). Because the root spacing is about 157, a 0.1 grid cannot miss a root.

- Number of modes: N=60 (λ60=9346).

- Coefficients: either numerical integration (relative tolerance 10−10) or the closed form of Section 7; the two agree.

- Evaluation: a vectorised sum over modes.

Convergence with the number of modes at r=0.09 m (temperature T in kelvin):

| t (s) | 5 modes | 10 modes | 20 modes | 60 modes | 200 modes |
| --- | --- | --- | --- | --- | --- |
| 2 | 303.9610 | 303.8618 | 303.9019 | 303.8976 | 303.8974 |
| 10 | 368.9965 | 368.9764 | 368.9845 | 368.9836 | 368.9836 |
| 40 | 446.98543 | 446.98538 | 446.98540 | 446.98540 | 446.98540 |

The truncation error at 60 modes is about 1.7×10−4 K at 2 s, 3×10−5 K at 10 s and ∼10−8 K at 40 s. That is about 3 % of the solver error (5.7×10−3 K at 2 s).

# 11. Verification of the solution itself (independent run)

Temperatures T at three radii (kelvin):

| t (s) | T(Ri)​ | T(0.09)​ | T(Ro)​ |
| --- | --- | --- | --- |
| 2 | 365.936 | 303.8976 | 300.098 |
| 10 | 472.933 | 368.9836 | 330.220 |
| 40 | 499.933 | 446.9854 | 402.198 |
| steady state | 500 | — | 405.68 |

- Differential equation at r=0.09 m, t=10 s, with fine central differences: θt=7.473 versus κ(θrr+θr/r)=7.435. The 0.5 % difference is within the noise of second-order finite differences with step 10−5 plus series truncation.

- Robin condition at Ro (t=10 s): kθr=−60555 and hθ=60441 (sum about 0.2 % of either term; the derivative is a one-sided first-order difference).

- Initial condition: θ(0.09,0)=0 exactly. Inner condition: θ(Ri,t)=f(t) exactly, because Rn(Ri)=0.

- Orthogonality (n=1,2): 2.3×10−16. Coefficients and norm: closed form equals numerical integral; Rn′(Ri)=−7.95775=−2/(πRi).

# 12. Map to the variables in the code

The code is LSTE_solver_R2_run_benchmark2.m.

| Mathematics | Name in the code |
| --- | --- |
| κ​ | kappa |
| β=1/t0​ | bD = 1/t0r |
| b, a, ψ​ | b_c, a_c, psi |
| Rn(r)​ | Rfun(l,r) |
| G(λ)​ | geig(l) |
| λn​ | lam (60 modes) |
| cn​ | cn (numerical integrals of numerator and denominator) |
| θ(r,t)​ | theta_exact(r,t) |

# 13. Eight-line summary for an appendix or a slide

- θt=κ(θrr+θr/r);  θ(Ri)=θ0(1−e−βt),  kθr+hθ=0 at Ro,  θ(0)=0.

- θ=f(t)ψ(r)+v with ψ=a+blnr (unit steady profile).

- vt=κ∇2v−f′ψ with homogeneous boundary conditions.

- Eigenfunctions: Rn=J0(λnr)Y0(λnRi)−Y0(λnr)J0(λnRi).

- Eigenvalues from the Robin condition: G(λn)=0.

- Orthogonality with weight r;  cn=−2/(πλn2Nn).

- vn+κλn2vn=−cnf′ ⇒ vn=−cnθ0β (e−βt−e−κλn2t)/(κλn2−β).

- θ=fψ+n​vnRn; with 60 modes the truncation error is ≤2×10−4 K.

# 14. Likely questions from the jury

- “Why is this solution exact?” It involves no spatial or temporal discretisation; the only approximation is truncating the series (error ≤2×10−4 K with 60 modes, table in Section 10). Its correctness was checked by confirming that it satisfies the equation, the boundary conditions and the initial condition (Section 11).

- “Why do the coefficients cn not decay?” Because ψ(Ri)=1 whereas Rn(Ri)=0; the limit is cn→−πRi/(Ro−Ri). The time factor 1/λn2 nevertheless makes the final series absolutely convergent.

- “Why an exponential ramp rather than a step?” Compatibility with the initial condition and better convergence. It is also the same type of load as the main problem (Tin=300+300(1−e−t/2)).

- “Does this solution also cover Lord–Shulman?” No, it is for Fourier conduction. For the Lord–Shulman wave, the time problem of each mode becomes a damped (second-order) oscillator, which is solved analytically in ls_wave_exact_R2.m.