# Abstract

Thermoelastic Analysis of Porous Graphene-Platelet-Reinforced Cylinders under Thermo-Mechanical Loading Using the Lord-Shulman Theory

In this thesis, the thermoelastic analysis of porous graphene-platelet-reinforced cylinders under thermo-mechanical loading is investigated using the Lord-Shulman theory. The hollow cylinders studied here are composed of layers whose properties vary through the thickness, this variation arising from the change in the volume fraction of the graphene platelets and the change in the porosity percentage in each layer.

In these hollow cylinders, the distributions of temperature, stresses, strains, and displacements are examined. To this end, the equations of motion and the Lord-Shulman energy-conservation equation are employed to derive the governing equations, boundary conditions, and compatibility conditions. Then, to discretize the governing equations together with the boundary and compatibility conditions, the efficient differential quadrature method is used in the spatial dimension and the Newmark method in the temporal dimension.

The elastic properties of the layers are computed by means of the Halpin-Tsai theory and the rule of mixtures, and to account for the change in properties due to porosity the open-cell (open-pore) theory is used.

Findings: Owing to the finite thermal-wave speed in the Lord-Shulman theory, the mid-wall temperature can overshoot the driving value (up to 1.115 at high graphene loading), and the temperature history shows the characteristic “peak-trough” pattern produced by wave reflection. Placing conductive graphene at the hot inner surface (pattern V) together with insulating porosity at the cold outer surface (pattern A) acts synergistically, reducing the outer-surface temperature by about 26-fold — the design configuration recommended by this work for thermal-protection applications. Under a Gaussian shock pulse, classical Fourier theory underestimates the peak temperature by about 2.5 times, a strong argument for the generalized theory. Methodologically, the differential-quadrature/Newmark combination reaches the desired accuracy at low computational cost, needing about one-fifteenth of the nodes required by the finite-difference and linear finite-element methods.

Conclusion: The results show that accurately analyzing transient thermal shock in these cylinders requires the generalized Lord-Shulman theory, since the classical theory severely underestimates transient stresses. From a design standpoint, aligning the graphene and porosity patterns (the V-A combination) gives the most favorable thermal behavior at the lowest stress penalty, though pattern selection must weigh both temperature and stress together. The layerwise differential quadrature method with Newmark integration provides an efficient, verified tool for such coupled thermoelastic analyses.

Keywords: thermoelastic analysis, porous cylinder, graphene platelets, thermo-mechanical loading, Lord-Shulman theory, differential quadrature method, Newmark method.